REUSE MANUAL

ELLIPSoid

10xxxxxx.1

Implementation

# Table of Contents

Table of Contents	i

SECTION 1.  Introduction	1

1.1	Purpose of the reuse Manual	1

1.2	PURPOSE OF THE REUSABLE SOFTWARE COMPONENT	1

1.3	GENERAL INFORMATION	1

1.3.1	POINT OF CONTACT	1

1.3.2	CERTIFICATION LEVEL	2

1.3.3	LEGAL RESTRICTIONS	2

Section 2.  INSTALLATION	3

2.1  PARTIAL REUSE	3

2.2  MODIFICATIONS	3

Section 3.  ENVIRONMENT	4

3.1	HARDWARE	4

3.1.1	DEVELOPMENT	4

3.1.2	TARGET	4

3.2	SOFTWARE	4

3.2.1	OPERATING SYSTEM	4

3.2.2	COMPILERS	4

3.3	ASSUMPTIONS AND PERFORMANCE LIMITATIONS	5

Section 4.  Global RSC Environment	6

4.1	TYPES	6

4.2	CONSTANTS	6

4.3	VARIABLES	6

4.4	INCLUDE FILES	6

4.5	dependencies	6

SECTION  5.  FUNCTIONS	8

5.1	Initialize_Ellipsoids	8

5.1.1	DESCRIPTION	8

5.1.2	INTERFACES AND EXAMPLES	8

5.1.3	DECLARATIONS	8

5.1.4	DEPENDENCIES	8

5.1.5	ERROR HANDLING	8

5.2	Create_ELLIPSOID	9

5.2.1	DESCRIPTION	9

5.2.2	INTERFACES AND EXAMPLES	9

5.2.3	DECLARATIONS	10

5.2.4	DEPENDENCIES	10

5.2.5	ERROR HANDLING	10

5.3	DELETE_ELLIPSOID	10

5.3.1	DESCRIPTION	10

5.3.2	INTERFACES AND EXAMPLES	11

5.3.3	DECLARATIONS	11

5.3.4	DEPENDENCIES	11

5.3.5	ERROR HANDLING	11

5.4	Ellipsoid_count	12

5.4.1	DESCRIPTION	12

5.4.2	INTERFACES AND EXAMPLES	12

5.4.3	DECLARATIONS	12

5.4.4	DEPENDENCIES	12

5.4.5	ERROR HANDLING	13

5.5	ellipsoid_index	13

5.5.1	DESCRIPTION	13

5.5.2	INTERFACES AND EXAMPLES	13

5.5.3	DECLARATIONS	13

5.5.4	DEPENDENCIES	14

5.5.5	ERROR HANDLING	14

5.6	ellipsoid_Name	14

5.6.1	DESCRIPTION	14

5.6.2	INTERFACES AND EXAMPLES	14

5.6.3	DECLARATIONS	15

5.6.4	DEPENDENCIES	15

5.6.5	ERROR HANDLING	15

5.7	ELLIPSOID_code	15

5.7.1	DESCRIPTION	15

5.7.2	INTERFACES AND EXAMPLES	16

5.7.3	DECLARATIONS	16

5.7.4	DEPENDENCIES	16

5.7.5	ERROR HANDLING	16

5.8	ellipsoid_PARAMETERS	17

5.8.1	DESCRIPTION	17

5.8.2	INTERFACES AND EXAMPLES	17

5.8.3	DECLARATIONS	17

5.8.4	DEPENDENCIES	18

5.8.5	ERROR HANDLING	18

5.9	ellipsoid_Eccentricity2	18

5.9.1	DESCRIPTION	18

5.9.2	INTERFACES AND EXAMPLES	18

5.9.3	DECLARATIONS	19

5.9.4	DEPENDENCIES	19

5.9.5	ERROR HANDLING	19

5.11	ellipsoid_USER_DEFINED	20

5.11.1	DESCRIPTION	20

5.11.2	INTERFACES AND EXAMPLES	20

5.11.3	DECLARATIONS	20

5.11.4	DEPENDENCIES	21

5.11.5	ERROR HANDLING	21

5.12	WGs84_PARAMETERS	21

5.12.1	DESCRIPTION	21

5.12.2	INTERFACES AND EXAMPLES	21

5.12.3	DECLARATIONS	21

5.12.4	DEPENDENCIES	22

5.12.5	ERROR HANDLING	22

5.13	WGS84_Eccentricity2	22

5.13.1	DESCRIPTION	22

5.13.2	INTERFACES AND EXAMPLES	22

5.13.3	DECLARATIONS	22

5.13.4	DEPENDENCIES	23

5.13.5	ERROR HANDLING	23

5.15	WGs72_PARAMETERS	23

5.15.1	DESCRIPTION	23

5.15.2	INTERFACES AND EXAMPLES	23

5.15.3	DECLARATIONS	24

5.15.4	DEPENDENCIES	24

5.15.5	ERROR HANDLING	24

5.16	WGS72_Eccentricity2	24

5.16.1	DESCRIPTION	24

5.16.2	INTERFACES AND EXAMPLES	24

5.16.3	DECLARATIONS	25

5.16.4	DEPENDENCIES	25

5.16.5	ERROR HANDLING	25

APPENDIX A  STRUCTURE/DEPENDENCY DIAGRAMS	26

APPENDIX B  DEFINITIONS/GLOSSARY	27

APPENDIX C  REFERENCES	28

# SECTION 1.  Introduction

## 1.1	Purpose of the reuse Manual

This document describes the characteristics of the ELLIPSOID reusable software component and provides instructions on its installation and operation.  The manual is a self-contained reference for the software engineer intending to incorporate the component in another software system.  This manual was written with the assumption that the user has a basic working knowledge of C and is familiar with fundamental C concepts and terminology.

## 1.2	PURPOSE OF THE REUSABLE SOFTWARE COMPONENT

The purpose of ELLIPSOID is to provide access to ellipsoid parameters for a collection of common ellipsoids.  A particular ellipsoid can be accessed by using its standard 2-letter code to find its index in the ellipsoid table.  The index can then be used to retrieve the ellipsoid name, parameters and determine if the ellipsoid is user defined.

By sequentially retrieving all of the ellipsoid codes and/or names, a menu of the available ellipsoids can be constructed.  The index values resulting from selections from this menu can then be used to access the parameters of the selected ellipsoid.

This component depends on a data file named “ellips.dat”, which contains the ellipsoid parameter values.  A copy of this file must be located in the directory specified by the value of the environment variable "GEOTRANS_DATA", if defined, or else in the current directory, whenever a program containing this component is executed.

Additional ellipsoids can be added to this file, either manually or using the Create_Ellipsoid function.  These ellipsoids are tagged in the file with an ‘*’.  However, if a large number of ellipsoids are added, the ellipsoid table array size in this component will have to be increased.  User defined ellipsoids can be deleted from the file using the Delete_Ellipsoid function.

This component depends on one other component:  the DATUM component to determine if a user defined ellipsoid is in use by a user defined datum.

## 1.3	GENERAL INFORMATION

### 1.3.1	POINT OF CONTACT

U.S. Army Topographic Engineering Center (USATEC)

Geospatial Information Division (GID)

ATTN:  CETEC-GD-A (Dan Specht)

7701 Telegraph Road

Alexandria, VA  22315-3864

Dan Specht		(703) 428 - 6761	Project Manager

### 1.3.2	CERTIFICATION LEVEL

This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

### 1.3.3	LEGAL RESTRICTIONS

This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# Section 2.  INSTALLATION

The following is a list of the files which make up the ELLIPSOID component:

Source Code Files:

ellipse.c

Header Files :

ellipse.h

Data Files :

ellips.dat

The compilation instructions for the ELLIPSOID component are as follows:

DOS Makefile (Uses Microsoft C):

cl /nologo /W3 /FR /G2 /DNDEBUG /Gs /Ox /AM /D_DOS /c ellipse.c

UNIX Makefile (Uses gcc compiler):

cc -g -O -ansi -Wall -c ellipse.c

The compilation order of the ELLIPSOID component relative to other components is unconstrained.

## 2.1  PARTIAL REUSE

The ELLIPSOID component does not allow for partial reuse.

## 2.2  MODIFICATIONS

The ELLIPSOID component does not permit modifications.

# Section 3.  ENVIRONMENT

This section provides details on the environment under which ELLIPSOID was developed, tested, and executed.

## 3.1	HARDWARE

### 3.1.1	DEVELOPMENT

The following is a list of hardware configurations under which ELLIPSOID was developed and tested.

- SUN SparcStation 20

- IBM compatible Pentium PC

### 3.1.2	TARGET

The following is a list of hardware configurations under which ELLIPSOID was executed.

- SUN SparcStation 20

- IBM compatible Pentium PC

## 3.2	SOFTWARE

### 3.2.1	OPERATING SYSTEM

The following is a list of operating systems under which ELLIPSOID was executed and tested.

- Solaris 2.7

- Windows 2000/NT

### 3.2.2	COMPILERS

The following is a list of compilers on which ELLIPSOID was compiled successfully.

- GCC version 2.95.2

- Microsoft Visual C++ version 6

## 3.3	ASSUMPTIONS AND PERFORMANCE LIMITATIONS

There are no hardware or environment constraints.  There are no limitations.

This RSC is written in ANSI C.

# Section 4.  Global RSC Environment

## 4.1	TYPES

Not applicable.  All types defined in this component are private.

## 4.2	CONSTANTS

The following is a list of significant visible constants declared globally in ELLIPSOID with their descriptions.

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_FILE_OPEN_ERROR		: Ellipsoid file opening error

ELLIPSE_INITIALIZE_ERROR		: Ellipsoid table can not initialize

ELLIPSE_TABLE_OVERFLOW_ERROR		: Ellipsoid table overflow

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid table not initialized properly

ELLIPSE_INVALID_INDEX_ERROR		: Index is an invalid value

ELLIPSE_INVALID_CODE_ERROR		: Code was not found in table

ELLIPSE_A_ERROR		: Semi-major axis less than or equal to zero

ELLIPSE_INV_F_ERROR		: Inverse flattening outside of valid range

(250 to 350)

ELLIPSE_IN_USE_ERROR		: User defined ellipsoid is in use by a user

defined datum

ELLIPSE_NOT_USERDEF_ERROR		: Ellipsoid not user defined - cannot be

deleted

## 4.3	VARIABLES

Not applicable.  All variables defined in this component are private.

## 4.4	INCLUDE FILES

ctype.h	: Standard C character handling library

stdio.h	: Standard C input/output library

stdlib.h	: Standard C general utility library

string.h	: Standard C string handling library

datum.h                          : Used to check if user defined ellipsoid is

in use by a user defined datum

ellipse.h	: Prototype error checking and error codes

## 4.5	dependencies

The following is a list of the software external to the RSC and its descriptions.

- ANSI C standard character handling, input/output, general utility, and string handling libraries are used as described above, and

- DATUM, which determines if a user defined ellipsoid is in use by a user defined datum.

# SECTION  5.  FUNCTIONS

## 5.1	Initialize_Ellipsoids

### 5.1.1	DESCRIPTION

This function initializes the module by reading “ellips.dat”, which must be in the directory specified by the value of the environment variable "GEOTRANS_DATA", if defined, or else in the current directory.  It uses the data from this file to create an ellipsoid table.

### 5.1.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Initialize_Ellipsoids();

Not applicable.

### 5.1.3	DECLARATIONS

#### 5.1.3.1	TYPES

Not applicable.

#### 5.1.3.2	CONSTANTS

Not applicable.

#### 5.1.3.3	VARIABLES

Not applicable.

### 5.1.4	DEPENDENCIES

- Assign_Ellipsoid_Row – used to copy a table entry.

### 5.1.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_FILE_OPEN_ERROR		: Ellipsoid file opening error

ELLIPSE_INITIALIZE_ERROR		: Ellipsoid table cannot initialize

ELLIPSE_TABLE_OVERFLOW_ERROR		: Ellipsoid table overflow

## 5.2	Create_ELLIPSOID

### 5.2.1	DESCRIPTION

This function creates a new ellipsoid with the specified code, name, and parameters, adds it to the ellipsoid table, and updates the ellipsoid data file.  The specified code must not already exist in the ellipsoid table.

### 5.2.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Create_Ellipsoid (const char *code,

- const char *name,

- double a,

- double f);

- code	The 2-letter code for the new ellipsoid (input),

- name	The name for the new ellipsoid (input),

- a	The semi-major axis, in meters, of the new ellipsoid (input),

- f	Flattening of the new ellipsoid (input).

Example:

- status = Create_Ellipsoid(code, name, a, f)

Inputs:

- code	"XX"

- name	"Experimental ellipsoid"

- a	6377295.664

- f	0.00332444929666288500

### 5.2.3	DECLARATIONS

#### 5.2.3.1	TYPES

Not applicable.

#### 5.2.3.2	CONSTANTS

Not applicable.

#### 5.2.3.3	VARIABLES

Not applicable.

### 5.2.4	DEPENDENCIES

None.

### 5.2.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid table not initialized properly

ELLIPSE_TABLE_OVERFLOW_ERROR		: Ellipsoid table overflow

ELLIPSE_INVALID_CODE_ERROR		: Code is already used in table

ELLIPSE_A_ERROR		: Semi-major axis less than or equal to zero

ELLIPSE_INV_F_ERROR		: Inverse flattening outside of valid range

(250 to 350)

ELLIPSE_FILE_OPEN_ERROR		: Ellipsoid file opening error

ELLIPSE_TABLE_OVERFLOW_ERROR		: Ellipsoid table overflow

## 5.3	DELETE_ELLIPSOID

### 5.3.1	DESCRIPTION

This function deletes a user defined ellipsoid with the specified code, deletes it from the ellipsoid table, and updates the ellipsoid data file.  The specified code must not be in use by a user defined datum.

### 5.3.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Delete_Ellipsoid (const char *code);

- code	The 2-letter code for the user defined ellipsoid (input)

Example:

- status = Delete_Ellipsoid(code)

Inputs:

- code	"XX"

### 5.3.3	DECLARATIONS

#### 5.3.3.1	TYPES

Not applicable.

#### 5.3.3.2	CONSTANTS

Not applicable.

#### 5.3.3.3	VARIABLES

Not applicable.

### 5.3.4	DEPENDENCIES

This function depends on Datum_Uses_Ellipsoid in the datum component to determine if a user defined ellipsoid is in use by a user defined datum.

### 5.3.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid table not initialized properly

ELLIPSE_IN_USE_ERROR		: User defined ellipsoid is in use by a user

defined datum

ELLIPSE_NOT_USERDEF_ERROR		: Ellipsoid is not user defined - cannot be

deleted

ELLIPSE_FILE_OPEN_ERROR		: Ellipsoid file opening error

## 5.4	Ellipsoid_count

### 5.4.1	DESCRIPTION

This function returns the number of ellipsoids in the ellipsoid table.

### 5.4.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Ellipsoid_Count (long *count);

- count	Number of ellipsoids in the ellipsoid table (output).

### 5.4.3	DECLARATIONS

#### 5.4.3.1	TYPES

Not applicable.

#### 5.4.3.2	CONSTANTS

Not applicable.

#### 5.4.3.3	VARIABLES

Not applicable.

### 5.4.4	DEPENDENCIES

None.

### 5.4.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid table not initialized properly

## 5.5	ellipsoid_index

### 5.5.1	DESCRIPTION

This function returns the index in the ellipsoid table of the ellipsoid with the specified 2-letter code.

### 5.5.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Ellipsoid_Index (const char *code,

- long *index);

- code	The 2-letter code for the desired ellipsoid (input),

- index	The index of the ellipsoid in the table with the specified code (output).

Example:

- status = Ellipsoid_Index(code, index)

Inputs:

- code	"ED"

Outputs:

- index	11

### 5.5.3	DECLARATIONS

#### 5.5.3.1	TYPES

Not applicable.

#### 5.5.3.2	CONSTANTS

Not applicable.

#### 5.5.3.3	VARIABLES

Not applicable.

### 5.5.4	DEPENDENCIES

None.

### 5.5.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid table not initialized properly

ELLIPSE_INVALID_CODE_ERROR		: Code was not found in table

## 5.6	ellipsoid_Name

### 5.6.1	DESCRIPTION

This function returns the name of the ellipsoid referenced by index.

### 5.6.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Ellipsoid_Name (const long index,

- char *name);

- index	The index of a given ellipsoid in the ellipsoid table (input),

- name	The name of the ellipsoid referenced by index (output).

Example:

- status = Ellipsoid_Name(index, name)

Inputs:

- index:	11

Outputs:

- name:	"Everest 1969 (West Malasia)"

### 5.6.3	DECLARATIONS

#### 5.6.3.1	TYPES

Not applicable.

#### 5.6.3.2	CONSTANTS

Not applicable.

#### 5.6.3.3	VARIABLES

Not applicable.

### 5.6.4	DEPENDENCIES

None.

### 5.6.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid table not initialized properly

ELLIPSE_INVALID_INDEX_ERROR		: Index is an invalid value

## 5.7	ELLIPSOID_code

### 5.7.1	DESCRIPTION

This function returns the 2-letter code of the ellipsoid referenced by index.

### 5.7.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Ellipsoid_Code (const long index,

- char *code);

- index	The index of a given ellipsoid in the ellipsoid table (input),

- code	The 2-letter code of the ellipsoid referenced by index (output).

### 5.7.3	DECLARATIONS

#### 5.7.3.1	TYPES

Not applicable.

#### 5.7.3.2	CONSTANTS

Not applicable.

#### 5.7.3.3	VARIABLES

Not applicable.

### 5.7.4	DEPENDENCIES

None.

### 5.7.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid table not initialized properly

ELLIPSE_INVALID_INDEX_ERROR		: Index is an invalid value

## 5.8	ellipsoid_PARAMETERS

### 5.8.1	DESCRIPTION

This function returns the semi-major axis and flattening of the ellipsoid referenced by the index.

### 5.8.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Ellipsoid_Parameters (const long index,

- double *a,

- double *f);

- index	The index of a given ellipsoid in the ellipsoid table (input),

- a	The semi-major axis, in meters, of the ellipsoid referenced by index (output),

- f	The flattening of the ellipsoid referenced by index (output).

Example:

- status = Ellipsoid_Parameters(index, a, f)

Inputs:

- index	11

Outputs:

- a	6377295.664

- f	0.00332444929666288500

### 5.8.3	DECLARATIONS

#### 5.8.3.1	TYPES

Not applicable.

#### 5.8.3.2	CONSTANTS

Not applicable.

#### 5.8.3.3	VARIABLES

Not applicable.

### 5.8.4	DEPENDENCIES

None.

### 5.8.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid module has not been initialized properly

ELLIPSE_INVALID_INDEX_ERROR		: Index out of valid range

## 5.9	ellipsoid_Eccentricity2

### 5.9.1	DESCRIPTION

This function returns the square of the eccentricity of the ellipsoid referenced by index.

### 5.9.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Ellipsoid_Eccentricity2 (const long index,

- double *e2);

- index	The index of a given ellipsoid in the ellipsoid table (input),

- e2	The square of the eccentricity of the ellipsoid referenced by index (output).

Example:

- status = Ellipsoid_Eccentricity2(index, e2)

Inputs:

- index	11

Outputs:

- e2	0.00663784660369520500

### 5.9.3	DECLARATIONS

#### 5.9.3.1	TYPES

Not applicable.

#### 5.9.3.2	CONSTANTS

Not applicable.

#### 5.9.3.3	VARIABLES

Not applicable.

### 5.9.4	DEPENDENCIES

None.

### 5.9.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid module has not been initialized properly

ELLIPSE_INVALID_INDEX_ERROR		: Index out of valid range

## 5.11	ellipsoid_USER_DEFINED

### 5.11.1	DESCRIPTION

This function checks whether or not the specified ellipsoid is user defined.  It returns 1 if the ellipsoid is user defined, and returns 0 otherwise.

### 5.11.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Ellipsoid_User_Defined (const long index,

- long *result);

- index	The index of a given ellipsoid in the ellipsoid table (input),

- result	Indicates whether the specified ellipsoid is user defined (1) or not (0) (output).

Example:

- status = Ellipsoid_User_Defined(index, *result)

Inputs:

- index	11

Outputs:

- result	1

### 5.11.3	DECLARATIONS

#### 5.11.3.1 TYPES

Not applicable.

#### 5.11.3.2 CONSTANTS

Not applicable.

#### 5.11.3.3 VARIABLES

Not applicable.

### 5.11.4	DEPENDENCIES

None.

### 5.11.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid module has not been initialized properly

ELLIPSE_INVALID_INDEX_ERROR		: Index out of valid range

## 5.12	WGs84_PARAMETERS

### 5.12.1	DESCRIPTION

This function returns the semi-major axis and flattening values of the WGS 84 ellipsoid.

### 5.12.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long WGS84_Parameters (double *a,

- double *f);

- a	The semi-major axis, in meters, of the WGS84 ellipsoid (output),

- f	The flattening of the WGS84 ellipsoid (output).

### 5.12.3	DECLARATIONS

#### 5.12.3.1 TYPES

Not applicable.

#### 5.12.3.2 CONSTANTS

Not applicable.

#### 5.12.3.3 VARIABLES

Not applicable.

### 5.12.4	DEPENDENCIES

None.

### 5.12.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid module has not been initialized properly

## 5.13	WGS84_Eccentricity2

### 5.13.1	DESCRIPTION

This function returns the square of the eccentricity of the WGS 84 ellipsoid.

### 5.13.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long WGS84_Eccentricity2 (double *e2);

- e2	The square of the eccentricity of the WGS84 ellipsoid (output).

### 5.13.3	DECLARATIONS

#### 5.13.3.1 TYPES

Not applicable.

#### 5.13.3.2 CONSTANTS

Not applicable.

#### 5.13.3.3 VARIABLES

Not applicable.

### 5.13.4	DEPENDENCIES

None.

### 5.13.5	ERROR HANDLING

This function checks that the table has been initialized.  The possible error codes are:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid module has not been initialized properly

ELLIPSE_INVALID_INDEX_ERROR		: Index out of valid range

## 5.15	WGs72_PARAMETERS

### 5.15.1	DESCRIPTION

This function returns the semi-major axis and flattening values of the WGS 72 ellipsoid.

### 5.15.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long WGS72_Parameters (double *a,

- double *f);

- a	The semi-major axis, in meters, of the WGS72 ellipsoid (output),

- f	The flattening of the WGS72 ellipsoid (output).

### 5.15.3	DECLARATIONS

#### 5.15.3.1 TYPES

Not applicable.

#### 5.15.3.2 CONSTANTS

Not applicable.

#### 5.15.3.3 VARIABLES

Not applicable.

### 5.15.4	DEPENDENCIES

None.

### 5.15.5	ERROR HANDLING

This function returns the following status codes:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid module has not been initialized properly

## 5.16	WGS72_Eccentricity2

### 5.16.1	DESCRIPTION

This function returns the square of the eccentricity of the WGS 72 ellipsoid.

### 5.16.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long WGS72_Eccentricity2 (double *e2);

- e2	The square of the eccentricity of the WGS72 ellipsoid (output).

### 5.16.3	DECLARATIONS

#### 5.16.3.1 TYPES

Not applicable.

#### 5.16.3.2 CONSTANTS

Not applicable.

#### 5.16.3.3 VARIABLES

Not applicable.

### 5.16.4	DEPENDENCIES

None.

### 5.16.5	ERROR HANDLING

This function checks that the table has been initialized.  The possible error codes are:

ELLIPSE_NO_ERROR		: No errors occurred in function

ELLIPSE_NOT_INITIALIZED_ERROR		: Ellipsoid module has not been initialized properly

ELLIPSE_INVALID_INDEX_ERROR		: Index out of valid range

# APPENDIX A  STRUCTURE/DEPENDENCY DIAGRAMS

This component consists of a single compilation unit and depends only on the standard ANSI C character handling, input/output, general utility, and string handling libraries, as well as on the DATUM component.

# APPENDIX B  DEFINITIONS/GLOSSARY

Eccentricity – The ratio of the distance between the center and a focus of an ellipse to the length of its semimajor axis.

Ellipsoid – The surface generated by an ellipse rotating about one of its axes.

Flattening – The ratio of the difference between the equatorial and polar radii of the Earth (semi-major and semi-minor axes) to its equatorial radius (semi-major axis).

Geoid – The equipotential surface of the earth’s gravity field approximated by undisturbed mean sea level of the oceans.  The direction of gravity passing through a given point on the geoid is perpendicular to this equipotential surface.

Geoid Separation – The distance between the geoid and the mathematical reference ellipsoid as measured along the ellipsoidal normal.  This distance is positive outside, or negative inside, the reference ellipsoid.  Also called geoidal height; undulation of the geoid.

Reference Ellipsoid – An ellipsoid whose dimensions closely approach the dimensions of the geoid;  the exact dimensions are determined by various considerations of the section of the earth’s surface concerned.  Usually a bi-axial ellipsoid of revolution.

Semi-Major Axis – One-half the longest diameter (equatorial axis) of an ellipsoid.

Semi-Minor Axis – One-half the shortest diameter (polar axis) of an ellipsoid.

WGS 72 – World Geodetic System 1972. WGS 72 was the previous DoD standard earth-centered, earth-fixed world geodetic system.  It was superceded by WGS 84.

WGS 84 – World Geodetic System 1984.  A world geodetic system provides the basic reference frame, geometric figure and gravimetric model for the earth, and provides the means for relating positions on various local geodetic systems to an earth-centered, earth-fixed coordinate system.  WGS 84 is the current DoD standard earth-centered, earth-fixed world geodetic system.

# APPENDIX C  REFERENCES

(1)  Topographic Engineering Center, TEC-SR-7, Handbook for transformation of DATUMS, PROJECTIONS, GRIDS, AND COMMON COORDINATE SYSTEMS, January 1996.

(2)  Department of Defense, MIL-HDBK-850, Military Handbook – Glossary of Mapping, Charting, and Geodetic Terms, 21 January 1994.